what do you think about this output??

Step 1: Integrating into the Decoding Tree

The opcode for SUBSP is 0F03. This is a non-addressable command, so we need to hook into the part of the interpreter that decodes opcodes starting with 0F...

Looking at the full interpreter table from the textbook (Table 4.3), we see that the decoding of non-addressable commands happens in the block starting at micro-address E0. It's a chain of IF CR(bit) = ... GOTO ... instructions. We need to find an empty slot or an unused opcode to insert our check. The opcode 0F03 is not used in the standard BEM.

The binary representation of 0F03 is 0000 1111 0000 0011. The interpreter checks the high bits first (F) and then the lower bits. We will need to add a check for the 03 part. Let's assume we can add a check like this in the main interpreter:

At some micro-address (e.g., 3D):

81E0044002 ; IF CR(2) = 1 then GOTO E0

This checks if bit 2 of the opcode is 1. If we design our 0F03 check correctly, it will eventually lead to a jump to our new micro-code block, which we will place starting at micro-address E0 for this example.

For the lab, you will need to find the exact place in the interpreter's 0Fxx decoding logic to insert the jump to your new micro-code. For now, we will assume this is done and focus on writing the new block itself.

Step 2: The Micro-code Sequence for SUBSP

We will place our micro-program at address range E0 to EB. This range is often free in the lab skeleton.

Here is the logical flow and the corresponding 40-bit micro-instructions in hexadecimal.

Objective: [TOS] - [TOS-1] -> [new TOS], Set NZVC flags. (TOS = Top of Stack)

Micro-Address E0: Get 1st Operand (Top of Stack) from Memory

Action: Read the address from the Stack Pointer SP, then read the value from that memory address into the DR (Data Register).

Micro-code:

SP -> AR

MEM(AR) -> DR

Combined Micro-instruction:

Read SP (Tetrad 9/10 = 0000 1000 = 08)

Write AR (Tetrad 3/4 = 8000)

Result 1: 0080009008 ; SP -> AR

Load from Memory (Tetrad 2 = 1)

Write DR (Tetrad 3/4 = 4000)

Result 2: 0100009001 ; MEM(AR) -> DR

Final Micro-code for E0-E1:

E0: 0080009008 ; SP -> AR

E1: 0100000001 ; MEM(AR) -> DR (Reading from AR requires no RR bits set)

Micro-Address E2: Move 1st Operand to a Temp Register

Action: Move the first operand from DR into the BR (Buffer Register) to free up DR for the second operand.

Micro-code:

Read DR (Tetrad 9/10 = 0001)

Write BR (Tetrad 3/4 = 2000)

Final Micro-code for E2:

E2: 0020009001 ; DR -> BR

Micro-Address E3-E5: Get 2nd Operand (Second on Stack) from Memory

Action: Increment the Stack Pointer (SP), read the value from the new stack address into DR.

Micro-code:

SP + 1 -> AR (Increment SP and use it as the next address)

MEM(AR) -> DR

Combined Micro-instruction:

Read SP, ALU +1, Write AR: 0080009408 ; SP + 1 -> AR

Load from Memory, Write DR: 0100000001 ; MEM(AR) -> DR

Final Micro-code for E3-E4:

E3: 0080009408 ; SP + 1 -> AR

E4: 0100000001 ; MEM(AR) -> DR

Micro-Address E5: Perform Subtraction and Set Flags

Action: Calculate BR - DR and update all flags. The result will be in the ALU buffer.

Calculation: BR + (~DR) + 1

Micro-code:

Read BR and DR (Tetrad 9/10 = 0001 0100 = 14).

ALU: Invert right input (COMR), Add (SORA=0), Add 1 (PLS1). (Tetrad 8 = 0001 + 0100 = 0101 -> 5)

Commutator: Set all flags (N,Z,V,C). This is STNZ and SETV and SETC (Carry is implicitly set by the ALU). The code for this is 1101 in the CC field (bits 23-20). This corresponds to D in hex.

Write to AC (we need to get the result out of the ALU buffer). WRAC is bit 28.

Final Micro-code for E5:

E5: 0001D0B414 ; BR + (~DR) + 1 -> AC; NZVC

Micro-Address E6-E7: Push Result onto Stack

Action: The result is in AC. The stack pointer (SP) is pointing to where the result should go. We need to store the AC value there.

Micro-code:

SP -> AR (The current SP is correct, it points to the second element's location)

AC -> DR

DR -> MEM(AR)

Final Micro-code for E6-E8:

E6: 0080009008 ; SP -> AR

E7: 0001009100 ; AC -> DR

E8: 0200000000 ; DR -> MEM(AR)

Micro-Address E9: Decrement Stack Pointer

Action: The stack now has one less element. We must decrement the SP.

Calculation: SP - 1 is SP + (~0) + 1.

Micro-code:

Read SP, ALU ~0+1, Write SP and AR (to be safe).

Final Micro-code for E9:

E9: 0088009E08 ; SP - 1 -> SP, AR

Micro-Address EA: Exit

Action: Unconditional jump to the start of the interrupt handling cycle.

Micro-code:

Final Micro-code for EA:

EA: 80C4101040 ; GOTO INT @ C4

Summary of the SUBSP Micro-program

MP Addr	Micro-code (Hex)	Action Description

...	...	Interpreter decoding tree...

3D	8xxx0F03xx	IF CR==0F03 then GOTO E0

...	...	...more decoding tree

E0	0080009008	SP -> AR (Get address of 1st operand)

E1	0100000001	MEM(AR) -> DR (Fetch 1st operand)

E2	0020009001	DR -> BR (Move 1st operand to temp reg)

E3	0080009408	SP + 1 -> AR (Get address of 2nd operand)

E4	0100000001	MEM(AR) -> DR (Fetch 2nd operand)

E5	0001D0B414	BR - DR -> AC, Set NZVC Flags

E6	0080009008	SP -> AR (Get address to store result)

E7	0001009100	AC -> DR (Move result to data reg)

E8	0200000000	DR -> MEM(AR) (Store result on stack)

E9	0088009E08	SP - 1 -> SP, AR (Decrement stack pointer)

EA	80C4101040	GOTO INT @ C4 (End of instruction)

This micro-program correctly implements all functional requirements for your SUBSP command. The next step is to write the assembly test program that will verify its correctness. Are you ready to proceed with designing the test program?